# Linux Public Driver Touchscreen Installation Instructions – For Serial Touchscreens

Elo TouchSystems is not responsible for usage, development, or support of public drivers and code.

Linux drivers for Elo touchscreens are available in the public domain, typically on Linux installation CDs and various FTP sites such as www.xfree86.org.  These Linux drivers are compatible with Elo AT4 2215, AccuTouch E271-2210, IntelliTouch E281-2310, IntelliTouch 2500S, and CarrollTouch 4000S controllers. The instructions listed below are for Red Hat Linux 7.1-7.3 and 8.0. Other versions of Linux including the newest versions may have different requirements. Elo TouchSystems is not responsible for the functionality of these drivers.

#### Driver Installation

1. Install Linux and verify that the X Windows System is working correctly.  We strongly recommend the system be configured to start in the command line mode and not be configured to start X Windows automatically.

2. Verify that the Elo touchscreen driver file, elographics_drv.o exists.  Typically, the file can be found at the following location: /usr/X11R6/lib/modules/input/elographics_drv.o.

#### For Redhat Linux 7.1 and 7.2
Edit the Configuration File

1. Edit the XF86Config-4 file, normally found in the /etc/X11 directory.  The instructions below are for a touchscreen connected to COM one.  In step three below, Option “Device” “/dev/ttyS0” specifies the COM port the touchscreen is connected to.  Use Option “Device” “/dev/ttyS0” for COM one, Option “Device” “/dev/ttyS1” for COM two, Option “Device” “/dev/ttyS2” for COM three, and Option “Device” “/dev/ttyS3” for COM four.

2. Add the following line to the “ServerLayout” Section:

Inputdevice  “touchscreen1” “SendCoreEvents”

3. Add the following section to the “Inputdevice” section:

Section “Inputdevice”
              Identifier “touchscreen1”
              Driver “elographics”
              Option “Device” “/dev/ttyS0”
              Option “AlwaysCore”
              Option “screenno” “0”

Option “MinimumXPosition” “600”
              Option “MaximumXPosition” “3600”
              Option “MinimumYPosition” “600”
              Option “MaximumYPosition” “3600”
              Option “UntouchDelay” “3”
              Option “ReportDelay” “1”
EndSection

4. Save changes to the XF86Config-4 file.

#### For Redhat Linux 7.3
Edit the Configuration File

1. Edit the XF86Config-4 file, normally found in the /etc/X11 directory.  The instructions below are for a touchscreen connected to COM one.  In step three below, Option “Device” “/dev/ttyS0” specifies the COM port the touchscreen is connected to.  Use Option “Device” “/dev/ttyS0” for COM one, Option “Device” “/dev/ttyS1” for COM two, Option “Device” “/dev/ttyS2” for COM three, and Option “Device” “/dev/ttyS3” for COM four.

2. Add the following line to the “ServerLayout” Section:

Inputdevice  “touchscreen1” “SendCoreEvents”

3. Add the following section to the “Inputdevice” section:

Section “Inputdevice”
              Identifier “touchscreen1”
              Driver “elographics”
              Option “Device” “/dev/ttyS0”
              Option “AlwaysCore”
              Option “screenno” “0”

Option “MinX”  “600”
              Option “MaxX”  “3600”
              Option “MinY”  “600”
              Option “MaxY”  “3600”
              Option “UntouchDelay” “3”
              Option “ReportDelay” “1”
EndSection

4. Save changes to the XF86Config-4 file.

#### For Redhat Linux 8.0
Edit the Configuration File

1. Edit the XF86Config file, normally found in the /etc/X11 directory.  The instructions below are for a touchscreen connected to COM one.  In step three below, Option “Device” “/dev/ttyS0” specifies the COM port the touchscreen is connected to.  Use Option “Device” “/dev/ttyS0” for COM one, Option “Device” “/dev/ttyS1” for COM two, Option “Device” “/dev/ttyS2” for COM three, and Option “Device” “/dev/ttyS3” for COM four.

2. Add the following line to the “ServerLayout” Section:

Inputdevice  “touchscreen1” “SendCoreEvents”

3. Add the following section to the “Inputdevice” section:

Section “Inputdevice”
              Identifier “touchscreen1”
              Driver “elographics”
              Option “Device” “/dev/ttyS0”
              Option “AlwaysCore”
              Option “screenno” “0”

Option “MinX”  “600”
              Option “MaxX”  “3600”
              Option “MinY”  “600”
              Option “MaxY”  “3600”
              Option “UntouchDelay” “3”
              Option “ReportDelay” “1”
EndSection

4. Save changes to the XF86Config file.

#### Restart X

1. If you configured your system to start X automatically, restart X by pressing Ctrl, Alt, Backspace.  If you configured your system to start in a command line mode, type “startx” at the command line prompt.  The touchmonitor should automatically be identified and the driver installed.  Complete the steps in the Touchscreen Calibration section for accurate video to touch alignment.

#### Touchscreen Calibration

1. See calibration utilities downloaded with this driver, or from www.elotouch.com.

#### Troubleshooting

1. To verify that the touchmonitor is connected correctly to the PC and that it is functioning correctly, type the following command in an X terminal window:

od -h -w10 </dev/ttyS0 — This is for COM1
od -h -w10 </dev/ttyS1 — This is for COM2

Touch the screen and you should see a group of numbers scrolling across the screen.

#### Screen Orientation Issues

Touchscreens are typically designed to be mounted to a display with a specific orientation so that calibration values will increase from left to right for the X-axis and from bottom to top for the Y-axis.  If the design of the touchscreen being used does not meet this criteria, is mounted so that it does not meet this criteria, or the touchscreen cable is connected backwards, cursor position will not match the position touched.  A typical symptom of this kind of problem after calibrating the touchscreen would be to touch one side of the touchscreen and the cursor would appear on the opposite side of the display.  The public driver does not include code to handle screen orientation issues.  To correct this problem, swap the MinX and MaxX values, and the MinY and MaxY values in the Inputdevice section of the Configuration file (Minimum/MaximumX/YPosition for earlier versions).

Elo TouchSystems is not responsible for usage, development, or support of public drivers and code.