Linux USB Public Driver Installation Instructions

NOTE: It has come to our attention that this driver does not support Linux kernel 2.6.11 or above.

Copy elousb_drv.o into the /usr/X11R6/lib/modules/input/ directory.

Find out which device your touchscreen uses with the following command:

cat /proc/bus/input/devices

It will give a result similar to the following for all your input devices (keyboards, mice, etc):

I: Bus=0003 Vendor=04e7 Product=0007 Version=0106

N: Name="Elo TouchSystems, Inc. Elo TouchSystems IntelliTouch 2500U"

P: Phys=usb-0000:00:1d.3-1/input0

H: Handlers=event4

B: EV=1b

B: KEY=10000 0 0 0 0 0 0 0 0

B: ABS=100 3

B: MSC=10

Handlers=event4 tell you which device to use. For this example, the device is /dev/input/event4. The example below uses event2 but yours may be different.

Add the following section to the X server configuration file as shown below:

Section "Inputdevice"

Identifier "TouchScreen"

Driver "elousb"

Option "Device" "/dev/input/event2"	# Check which /dev/input/eventX your touchscreen uses.

Option "MinX" "4095"

Option "MaxX" "0"

Option "MinY" "4095"

Option "MaxY" "0"

Option "ScreenNumber" "1"

Option "AlwaysCore" "On"

EndSection

Add the following to the ServerLayout section:

Inputdevice "TouchScreen"

#### Touchscreen Calibration

1. See calibration utilities downloaded with this driver, or from www.elotouch.com.