# Linux Public Driver Calibration Instructions

The calibration utility is designed to work with Elo TouchSystems serial touchscreen controllers and Linux public serial drivers. The calibration utility requires Motif version 2.2.3 or later be installed.

Linux drivers for Elo touchscreens are available in the public domain, typically on Linux installation CDs, various FTP sites such as www.xfree86.org and at www.elotouch.com/support/dnld.asp#linux. The instructions listed below are for Red Hat Linux 9.0. Other versions of Linux including the newest versions may have different requirements. Elo TouchSystems is not responsible for the functionality of these drivers.

For Redhat Linux 9.0

Create a folder for the Elo Calibration utility. Copy the utility, file name eloXCalibration_100405.tgz to that directory and untar the file there.

Open a terminal window and go to the folder containing the Elo Calibration utility.

Type the following to run the calibration utility:

./elocalX

Follow the instructions listed on the screen.

If you have the touchscreen connected to a COM port other than COM 1 or COM 2 type the following at a command prompt to run the calibration utility. In the example below COM 4 is being used.

./elocalx –newdev /dev/ttyS3 –speed B9600 –parms 8N1

After successfully calibrating the touchscreen, the results will be shown on the display under the heading,  Section “InputDevice.”

Copy the complete result to the InputDevice section of the appropriate configuration file for your system. For Red Hat 9.0 the configuration file name is XF86Config.

Add the following line to the ServerLayout section of the appropriate configuration file for your system. For Red Hat 9.0 the configuration file name is XF86Config.

Inputdevice  “touchscreen1” “SendCoreEvents”

Save changes to the configuration file and reboot the X server by pressing Ctrl, Alt, Backspace or restart the computer.

Verify the touchscreen is calibrated and working properly. If it isn’t please repeat the steps listed above.

#### Contacting Elo TouchSystems

Please contact technical support for assistance.

#### Additional Information

The syntax for the elocalX command is:

elocalX  [X-args] [-help]

[-full-screen | -max-window ]

[-timeout Nsecs ]

[-debug-out Nd ]

[-accuracy Npixel ]

[-input-dev devname [-speed value -parms value]]

[[-newdev  devname [-speed value -parms value]]  .... ]

where :

X-args	optional arguments passed to X (e.g., -geometry, -bg)

-help	prints usage information

-full-screen	open the calibration window with dimensions equal to screen dimensions (default mode)

-max-window	open the calibration window with maximum window dimensions allowed by window manager

-timeout	the number of seconds for waiting the touching the screen (default – 30 secs)

-debug-out	debug messaging level (default 0 – no debug messages; 2 – all debug messages output to stderr)

-accuracy	the number of touchscreen pixels allowed for difference between coordinates for toched corners (default 32)

-input-dev	define the input device instead use a devices list

-newdev	signals the start of a new device definition

-speed	speed for a serial port in the form B<baudrate> or <baudrate>

-parms	8N1    parameters for serial port in the form <databits>[N|E|O]<stopbits>

Examples of invocation include:

Start elocalX and add the USB device name /dev/input/event8 to the dropdown list for choice of touchscreen input type.  Set “ELO Calibration” as the window title.

elocalX -title "ELO Calibration" –newdev /dev/input/event8

Start elocalX and add the serial device name /dev/ttyS2 to the dropdown list for choice of touchscreen input type.

elocalX -newdev /dev/ttyS2 -speed B19200 -parms 8N1

Start elocalX with read data from USB touchscreen /dev/input/event2, with timeout 10 secs, and drop results to the file eloUSB:

elocalX –input-dev /dev/input/event2 –timeout 10 >eloUSB &

elocalX Behavior

There are some guidelines to using elocalX, which include:

- The user must remove the Elo Touchscreen entries from the X configuration file (/etc/X11/XF86Config or /etc/X11/xorg.conf) before using elocalX.  These configuration values will be computed by elocalX.

- The user must login as ‘root’  for correct results.

elocalX operates in the following way:

- Displays a window with three X-shaped cross-hairs in the corners of a “full-screen-size” window without window decorating.

- Gives instructions to the user on a message line near the bottom of the screen, and the progress bar for waiting the screen touch.

- If you use the –input-dev parameter, the program will not display a list of common input devices in the drop-down box.

- If the input device opens correctly, the user is asked to touch each of the three cross-hairs.  Otherwise, a message is sent to stderr indicating that the input device cannot be opened.

- If user doesn’t touch the screen within 30 secs (or the -timeout value), the application finishes without output to the result file.

- The program monitors to see if you have pressed the same Y coordinate for the two top cross-hairs with a precision of plus-or-minus -accuracy pixels.  The program also monitors to ensure that the user has pressed within four pixels for X coordinates, as well.  If the user doesn’t do this, he is asked to restart the application.

- After the user has pressed all three touchpoints, the result is printed on the console to stdout, which can be redirected to a file from the command line.

- The user can then use the result file to edit the X configuration file, or use the edit script (described below).

xf86cfgedit.sh

The xf86cfgedit.sh script can be used to edit the X Configuration file (/etc/X11/XF86Config or /etc/X11/xorg.conf) with elocalX results.

The syntax for the xf86cfgedit.sh command is:

Xf86cfgedit.sh X-conf-file elocalX-file

where :

X-conf-file		name of X Configuration file: /etc/X11/XF86Config, or /etc/X11/xorg.conf, or a copy of one

elocalX-file	stdout file from elocalX command

The Xf86cfgedit.sh script operates as follows:

- tries to find the sections ServerLayout and InputDevice in the X configuration file X-conf-file

- copies the original X-conf-file to the file with the name X-conf-file.process# , and prints a message that it has done that (for restoring the original values)

- if sections are found, the file elocalX-file is inserted after the last InputDevice section, and a line is inserted in the ServerLayout section

Build Instructions

The tarfile de-archived into any desired directory.  At that point, the makefile, Makefile.elocalX can be used to build the elocalX executable.