TTP 7020/7030 Printer FW version 3.00

Release date: 18 May 2005

This firmware release includes the features of the previous TTP 7020/7030 firmware releases. It is for use with the following TTP 7000 printer models:

| Printer Model | Firmware Header |
| --- | --- |
| TTP 7020 | 01756-xxx |
| TTP 7030 | 01756-xxx |
| Example: 01756-300 means version 3.00 |  |

Installation

- Download and install the Toolbox utility program which is available on www.zebra.com.

- Start the Toolbox utility program.

- Select printer type and port accordingly.

- Select Tools, Upload file to printer.

- In the File Type window that appears, select Firmware.

- Press the Add button and add the firmware file you want to load to the File to Upload list.

- When ready, select the firmware file you just added to the list and click on Begin Upload.

- Wait until after the printer buzzes to indicate firmware loading is completed.

- Print a self-test page to verify that the printer has the new firmware.

New Features

- The printer will now retry three times (cut + clear presenter) when failing to clear the presenter.

- Added 1 second delay before Autoload starts to avoid paper jam and limit noise at start of Autoload.

- Parameter n36 (Document mode) extended with “2-Black mark mode”.

- Parameter n49 defaults to 1.

- Parameter n56 (Max status code) added (default value: 255).

- Print buffer handling speed increased.

- nFault (parallel port) will now be set on all types of general faults.

- ESC P + unknown will terminate ESC P command sequence.

- Adding an ACK-marker will now force printout.

Issues Corrected

- Parallel and USB IRQ are only enabled when detected

- Improved speed compensation when using long burn times and high speed.

- The error state clean up process will make sure that a found ACK-marker will be answered before deleted.

- The printer will no longer report PrintDataInBuffer when it is in TestMode so that the driver will not try to clear it up and cause two-three extra empty receipts.

- Fixed when the Feed Button stops responding.

- Fixed when ESC ENQ 06h reported a pending error but ESC ENQ 01 indicates “No error”.

- The FF button will now work regardless of how parameter n34 (AutoCut after FF) is set.

- The test receipt now prints parameter n49 correctly.