I/F-Com

Windows 2000 / XP Thermal Printer Driver Manual

Rev 1.0

Content

Installing the driver	3

Operating system	3

Files needed	3

There are three ways of installing the driver	3

Important	3

Using INSTALLER.EXE	4

The welcome screen	4

License agreement	4

Operation	5

Interface and Printer	6

Port	6

Summary	7

Using QUICKINSTALL.EXE	8

Using the Add Printer Wizard	9

The welcome screen	9

Local or Network Printer	9

Port	10

Install Printer Software	10

Locate File	11

Select Model	11

Name your printer	12

Printer Sharing	12

Test Page	13

Summary	13

Windows Logo Testing	14

Settings printer properties	15

The welcome screen	15

Basic Driver Information	15

Paper Settings	16

Cutter Settings	16

Basic Printer Settings	17

Advanced Settings	18

Compensation Settings	19

Summary	19

Preferences	20

Problems when printing	20

General guidelines	20

Making screen dumps	20

If the driver is not printing at all	21

If the printout is garbled	21

Reading status from a bi-directional printer	22

The code	22

The 4 byte status	22

## Installing the driver

### Operating system

The driver supports the following operating systems

Windows 2000 ( all service packs )

Windows XP ( all versions, all service packs )

Windows XP embedded

### Files needed

The driver package consists of the following files, please make sure that all file exists in the folder where the driver is installed from

Files marked with *) MUST be present

Files marked with **) MUST be present if installing a board with bi-directional support

IFC_DRV.DLL		*)

IFC_UI.DLL			*)

BASICS.INI			*)

IFC_DRV.INI		*)

HELP.INI

BOARDS.INI

PRINTERS.INI

INSTALLER.EXE

PROPERTIES.EXE

UPDATER.EXE

QUICKINSTALL:EXE

DIAGNOSTICS.EXE

IFCOM.INF

IFCOM BIDIRECTIONAL.INF

IFC_LM.DLL		**)

### There are three ways of installing the driver

1)

Using the INSTALLER.EXE application provided with the driver

This will step by step guide you through the set-up, selecting interface board and printer attached to your system

This is the preferred method

The driver will be ready for use immediately.

2)

Using the QUICKINSTALL.EXE

This method will install the driver with an absolute minimum of user intervention.

Only use this method if you have received a specialised driver package

The driver will be ready for use immediately

3)

Using the normal Windows Add Printer wizard

Only use this approach if you have received a specialised driver package, or if you are installing on an embedded system.

The driver might need additional setting up before being ready for use.

### Important

Do NOT mix the various methods when installing, updating or uninstalling the driver.

## Using INSTALLER.EXE

Launch installer.exe and proceed through the various steps.

### The welcome screen

The welcome screen shows the basic information, including the package relese and the driver version number.

### License agreement

Read and accept the license agreement

### Operation

Select the operation to perform

The installer will detect what options are available

Fresh Install

No previous version of the driver was detected. The driver will be installed from scratch

Check for updates ( This option might not be available for editing )

The installer will check the internet for new driver versions

Update

The currently installed driver will be updated with the new driver

Revert

The currently installed driver is newer. Selecting this option will revert the driver back to an older version.

Uninstall

The driver will be uninstalled

Keep Settings

If checking this checkbox, the current settings will be saved. When installing a new driver, these settings will be used.

### Interface and Printer

Select the interface board and printer attached to your computer

It is important that these settings matches the actual hardware, otherwise the driver might not function properly

### Port

Select the port attached to the interface board.

When using COM ports, please make sure the port settings matches those of the interface board.

Normally the settings are

Baudrate	115200

Databits	8

Parity	None

Stopbits	1

Handshake	Hardware

OBS

When installing a board using USB, on a computer where there previously has never been an USB device attached, no USB port will be listed.

To avoid this situation, please connect the printer board to the computer on any USB port, and power it up. This will result in Windows installing the needed components to start using the interface board on USB.

Bi-directional Support ( This option might not be available for editing )

When checked, the driver supports bi-directional data transfers, enabling reading status, temperatures and voltages from the interface board.

Edit advanced settings ( This option might not be available for editing )

When checked, additional advanced settings will be available

### Summary

The last page will show the selected settings. Please review and make sure they match your hardware.

When clicking Finish, the driver will be installed, updated or uninstalled, according to selection.

Please be patient, the operation might take a few minutes.

You might be prompted to reboot Windows after successful installation.

## Using QUICKINSTALL.EXE

The quickinstaller will install the driver without allowing the user to make any selections.

This is use ONLY for drivers made for a specific hardware environment.

The quickinstaller can also be set up to execute automatically without the user pressing Install first.

## Using the Add Printer Wizard

From Start-Settings select Printers and Faxes

Click on Add Printer

### The welcome screen

On the welcome screen click next

### Local or Network Printer

Select local printer and uncheck Automatic detect.

### Port

Select the port attached to the printer hardware

### Install Printer Software

When prompted for what type of printer to install, click on Have Disk

### Locate File

Navigate to the folder where the driver package is located, and select the file IFCOM.INF

Open the file and click OK

### Select Model

You will be presented with two choices

Choice named (bidir)  will support bi-directional printing, enabling reading status, temperatures and voltages from the interface board.

Select the driver that matches your needs

Selecting a bidir driver on a non-bidir interface board, could result in applications or utilities using the driver, being sluggish or unresponsive.

### Name your printer

Select a printer name ( normally there will be no need to change the default name ), and select if the printer should be used as default printer

### Printer Sharing

Select if the printer should be shared ( normally this will NOT be the case )

### Test Page

Select if you want a testpage to be written when the installation is complete

OBS

Using this approach, the printer driver might need additional setting up before working properly, so it is strongly recommended that you do NOT print a testpage at this point

### Summary

Review the settings, and if correct click Finish to actually install the printer driver

### Windows Logo Testing

You will be notified that the driver is not signed, please select Continue Anyway.

Upon successful installation, the Add Printer Wizard will simply close.

The I/F-Com printer driver support ALL interface boards and printers, and as this is a constantly growing number, the printer driver continues to be expanded and improved.

Because of this I/F-Com have chosen NOT to certify the driver, as the certification would be obsolete as soon as the driver was updated.

The printer can now be accessed from Start-Settings-Printers and Faxes

## Settings printer properties

### The welcome screen

The welcome screen shows the basic information, including the package relese and the driver version number.

### Basic Driver Information

The Basic Driver Information shows the current driver settings. These should match your hardware.

If driver settings for some reason have been mixed up, all settings can be restored to factory defaults by checking Restore all settings, and proceeding.

If experiencing problems with the driver, Debug Information can be checked.

Under normal circumstances, make sure that this checkbox is NOT checked.

### Paper Settings

The Paper Settings reflects the paper size used when printing from page sized applications such as Word or WordPad.

Make sure that these settings match any settings of the application ( such as Word ) otherwise a warning will be issued when printing.

### Cutter Settings

If the hardware has a cutter installed, the functionality of the cutter can be set here.

The Cutter Settings also concludes the settings that has to do with the most basic settings. To edit the more advanced features, check Edit Advanced Settings and click next

### Basic Printer Settings

The Basic Printer Settings defines the most basic of the advanced settings

The command set defines the actual commands send to the board. Most interface boards will understand the Basic command set, but in order for more advanced features to work, the Extended command set has to be used

Speed Settings defined the maximum printing speed, the Fallback Speed defines the speed the printer falls back to if maximum speed can not be obtained. ( These settings might not be available for editing )

Burn Time defines the default time the thermal head is activated. ( This setting might not be available for editing )

Data compression defines how graphics data is sent to the interface board. If in doubt what to use, most boards will understand IFC Compressed.

The compression level indicates how much data are compressed. If set to None, data is compressed without loss, if set to High, data is compressed as much as possible, even at the expense of loosing information.

### Advanced Settings

Advanced settings should only be altered is the user fully understands the implications of the changes.

Peak limitation limits the max allowable power drawn by the thermal head, the higher the peak limitation, the less power will be drawn from the power supply. Using power limitation limits the maximum printing speed

Burn correction corrects the time power is applied to the thermal head.

Adjust if printout seems to light or to dark,

Top correction improves quality on white to black transitions on low quality papers.

Note, that using top correction effectively reduces the maximum printing speed.

### Compensation Settings

Compensation is for technical user only, and is merely intended for laboratory use in case of setting up a printer for use under very varying conditions, or under conditions not covered by the basic compensation build into the firmware.

Please contact I/F-Com for guidelines on using Compensation

### Summary

Review the settings

### Preferences

When printing, an extra set-up page is available for selecting the basic printing preferences

Copies selects the number of copies to print

Print Quality selects the basic print quality

Halftone selects the halftone scheme when printing greyscales

( One, more or all of these features might be unavailable for editing, depending on the driver build )

## Problems when printing

### General guidelines

If encountering problems while printing please check the following.

The most important thing is to make sure the interface board is connected to the correct port, and in case of a COM port, that the port settings are correct.

Normally a COM port uses the standard

Baudrate	115200

Databits	8

Parity	N

Stopbits	1

Handshake	Hardware

### Making screen dumps

Before proceeding, launch the application PROPERTIES.EXE, and on the second page “Basic Driver Information”, make sure the checkbox “Dump Printout” is checked.

When the printer driver is working satisfactory, clear this checkbox again.

First print a testpage, and then locate the file C:\DUMP_PAGE_1.BMP

The file is a standard bitmap representation of the output, and should look correct.

If it does not, try to restart your computer and try again.

If it still does not look like expected, uninstall the driver, restart and install the driver again.

The width of the bitmap should match the dotwidth of your printer.

If it does not, make sure the selected interface board and printer matches your hardware.

When the bitmap looks correct, proceed to the next step.

### If the driver is not printing at all

1)

The most likely cause is a hardware problem.

Recheck connections and port settings.

### If the printout is garbled

1)

Launch PROPERTIES.EXE and go the the page “Basic Printer Settings”

Select Data compression = Basic

Try both Standard and Extended Command set

2)

Launch PROPERTIES.EXE and go the the page “Advanced Settings”

Make sure everything is disabled

Go to the page “Compensation Settings”

Make sure everything is disabled

At this point, your printer should be printing

## Reading status from a bi-directional printer

### The code

Reading the status is done as shown below, note that error checking etc is omitted for clarity

// data used

HANDLE 			hPrinter;

PRINTER_DEFAULTS		Defaults;

BYTE			bBuffer[ 4 ];

DWORD			dwReceived;

// set printer defaults

Defaults = { ””, NULL, PRINTER_ALL_ACCESS };

// start by opening the printer

// note that the printer name might be “Ifcom Thermal Printer (bidir)”

OpenPrinter( “Ifcom Thermal Printer”, &hPrinter, &Defaults );

// read the 4 byte status

GetPrinterData( hPrinter, "IFCom_Get_Status", NULL, bBuffer, 4, &dwReceived );

// close the printer

ClosePrinter( hPrinter );

### The 4 byte status

Byte 0	Status

Byte 1	Head temperature ( 0 – 127 )

Byte 2	Voltage ( 0 – 127 )

Byte 3	Motor temperature ( 0 – 127 ) ( reads 0 if not implemented )

Temperatures and voltages are returned as a relative reading

Status

D7	Always 1

D6	Receive buffer full

D5	Receive buffer overrun

D4	Cutter error

D3	Head up

D2	Temperature shut down

D1	Paper out

D0	Paper near end

Notice

At this point status can only be retrieved via serial ports.

Not all interface boards might supports reading the status.