| Basic Setup |  |  |  |
| --- | --- | --- | --- |
| 1 | AT | Attention command | Support |
| 2 | ATZ | Modem reset | Support |
| 3 | AT&F | Restore to factory default settings | Support |
| 4 | ATI | Identification information | Support |
| 5 | ATE | Command echo | Support |
| 6 | ATQ | Result code suppression | Support |
| 7 | ATV | Response mode | Support |
| 8 | ATX | Call progress monitor | Support |
| 9 | ATS0 | Auto answer | Support |
| 10 | ATS2 | Escape sequence character | Support |
| 11 | ATS6 | Dialing delay control | Support |
| 12 | ATS7 | Connection complete timeout | Support |
| 13 | ATS8 | Comma dial modifier delay control | Support |
| 14 | ATS10 | Automatic disconnect delay control | Support |
| Interface Setup |  |  |  |
| 15 | AT&C | DCD control | Support |
| 16 | AT&D | DTR control | Support |
| 17 | AT+IFC | DTE flow control | Support |
| 18 | AT+IPR | DTE data rate | Support |
| 19 | AT+ICF | Character format | Not Support |
| 20 | AT+ILRR | Data rate reporting | Not Support |
| Phone Identification |  |  |  |
| 21 | AT+CGMI | Manufacturer identification | Support |
| 22 | AT+CGMM | Model identification | Support |
| 23 | AT+CGMR | Revision identification | Support |
| 24 | AT+CGSN | IMEI identification | Support |
| Voice Call Basic |  |  |  |
| 25 | ATDnum | Voice dailing | Not Support |
| 26 | ATA | Answer incoming voice call | Not Support |

| 27 | ATH_ | (Hangup_ call) Hangup voice call | Not Support |
| --- | --- | --- | --- |
| 28 | ATH_ | (Reject_ call) Reject voice call before answer | Not Support |
| 29 | ATD>entry | Dial entry in current phonebook | Not Support |
| 30 | ATDL | Dial last number | Partial |
| Enhanced Call Features |  |  |  |
| 31 | AT+CLIP | Caller line identification control | Not Support |
| 32 | AT+CCWA | Call waiting control | Not Support |
| 33 | AT+CLCC | List current calls and status | Not Support |
| 34 | AT+CCFC | Call forwarding | Not Support |
| Phone Status and Control |  |  |  |
| 35 | AT+CSCS | Select TE character set | Support |
| 36 | AT+CBC | Battery charge | Support |
| 37 | AT+CPAS | Phone activity status | Not Support |
| 38 | AT+CFUN | Set phone functionality (RF enable/disable) | Not Support |
| 39 | AT+CHLD | Hold active line / pick second line / retrieve held line | Not Support |
| 40 | AT+CLCK | Facility lock | Not Support |
| 41 | AT+CPIN | Enter password | Support |
| 42 | AT+CPWD | Change password | Support |
| 43 | AT+VTS | DTMF generation during voice call | Not Support |
| 44 | AT+VTD | Define DTMF tone duration | Not Support |
| Call Meter Related |  |  |  |
| 45 | AT+CACM | Accumulated call meter | Not Support |
| 46 | AT+CAMM | Accumulated call meter maximum | Not Support |
| 47 | AT+CPUC | Price per unit | Not Support |
| 48 | AT+CAOC | Advice of call charge | Not Support |
| Audio Control |  |  |  |
| 49 | AT+CMUT | Mute control | Support |
| 50 | AT+CLVL | Speaker volume level | Not Support |
| SMS Handling |  |  |  |
| 51 | AT+CSMS | Select SMS service | Support |

| 52 | AT+CPMS | Select message storeage |  | Support |
| --- | --- | --- | --- | --- |
| 53 | AT+CMGD | Delete message |  | Support |
| 54 | AT+CSCA | Service center address |  | Support |
| 55 | AT+CMGF | Select message format (0 = PDU) |  | Partial |
| 56 | AT+CMGL | List messages (PDU mode) |  | Partial |
| 57 | AT+CMGR | Read message (PDU mode) |  | Partial |
| 58 | AT+CMGS | Send message direct (PDU mode) |  | Not Support |
| 59 | AT+CNMI | New message indication |  | Not Support |
| 60 | AT+CSCB | Select cell broadcast message |  | Not Support |
| Phonebook Handling |  |  |  |  |
| 61 | AT+CPBW | Write phonebook entry |  | Support |
| 62 | AT+CPBS | Select phonebook storage |  | Support |
| 63 | AT+CPBF | Find phone book entries |  | Partial |
| 64 | AT+CPBR | Read phonebook entries |  | Support |
| Operator and Subscriber Related |  |  |  |  |
| 65 | AT+CSQ | Signal quality |  | Support |
| 66 | AT+COPS | Operator selection |  | Partial |
| 67 | AT+CREG | Network registration |  | Support |
| 68 | AT+CNUM | Subscriber identity |  | Not Support |
| CSD Data Call |  |  |  |  |
| 69 | ATDnum_ | (CSD_ data_ call) GSM CSD data call |  | Not Support |
| 70 | +++ | Escape sequence from data to command mode |  | Not Support |
| 71 | ATO | Return to data mode from command mode |  | Not Support |
| 72 | ATH_ | (CSD_ data_ call) Disconnect CSD data call |  | Not Support |
| GPRS Data Call |  |  |  |  |
| 73 | AT+CGDCONT |  | GPRS define PDP context | Support |
| 74 | AT+CGQREQ |  | GPRS QoS (request) | Support |
| 75 | AT+CGQMIN |  | GPRS QoS (minimum accepted) | Support |
| 76 | AT+CGATT |  | GPRS attach | Support |
| 77 | AT+CGACT |  | GPRS activate PDP context | Support |
| 78 | ATD_ |  | (GPRS_ data_ call) GPRS data connect | Support |

| 79 | AT+CGREG | GPRS network registration | Not Support |
| --- | --- | --- | --- |
| 80 | +++ | Escape sequence from data to command mode | Not Support |
| 81 | ATO | Return to data mode from command mode | Not Support |
| 82 | ATH_ | (GPRS_ data_ call) Disconnect GPRS data call | Not Support |
| 83 | AT+CGEREP | GPRS event reporting | Not Support |
| Status, Events and Error Reporting |  |  |  |
| 84 | AT+CR | Service reporting control | Support |
| 85 | AT+CRC | Cellular result codes control | Support |
| 86 | AT+CMEE | Extended error report | Support |
| FAX |  |  |  |
| 87 | AT+ FCLASS | FAX: select, read or test service class | Support |
| 88 | AT+ FMI | FAX: Report manufacturer ID | Support |
| 89 | AT+ FMM | FAX: report model ID | Support |
| 90 | AT+ FMR | FAX: report revision | Support |
| 91 | AT+ FTS | FAX: Stop transmission & wait | Not Support |
| 92 | AT+ FRS | FAX: Wait for silence | Not Support |
| 93 | AT+ FTM | FAX: transmit data with carrier | Not Support |
| 94 | AT+ FRM | FAX: receive data with carrier | Not Support |
| 95 | AT+ FTH | Transmit HDLC data with carrier | Not Support |
| 96 | AT+ FRH | Receive HDLC data with carrier | Not Support |
| 97 | AT+ FDT | Transmit Data | Not Support |
| 98 | AT+ FDR | Receiver Data | Not Support |
| 99 | AT+ FET | Transmit page punctuation | Not Support |
| 100 | AT+ FPTS | Page transfer status parameters | Not Support |
| 101 | AT+ FK | Terminate Session | Not Support |
| 102 | AT+ FBOR | Page transfer bit order | Not Support |
| 103 | AT+ FBUF | Buffer size report | Not Support |
| 104 | AT+ FCQ | Copy quality checking | Not Support |
| 105 | AT+ FCR | Capability to receive | Not Support |
| 106 | AT+ FDIS | Current sessions parameters | Not Support |

| 107 | AT+ FDCC | DCE capabilities parameters | Not Support |
| --- | --- | --- | --- |
| 108 | AT+ FLID | Local ID String | Not Support |
| 109 | AT+ FPHCTO | Page transfer timout paramerts | Not Support |

Please see the following sample for sending and receiving SMS.

select message storage
 
at+cpms=?         
+CPMS:(IM,OM,BM,MT,DM),(OM,DM),(IM)                                   
OK  
at+cpms="MT"            
+CPMS:2,352           
OK  
 
select character set
 
1) if you want to send message in English 
 
at+cscs=?
+CSCS: ("8859-1","ASCII","GSM","UCS2","UTF8")
OK
at+cscs="ASCII"
OK
 
Write SMS message to memory
 
at+cmgw="0928575285"
> Test message 1
><ctrl-z>
+CMGW: 112
 
OK
at+cmss=112
+CMSS: 5
OK
 
2) If you want to send message in Chinese
 
at+cscs=?
+CSCS: ("8859-1","ASCII","GSM","UCS2","UTF8")
OK
at+cscs="UCS2"
OK
 
at+cmgw="0928575285"
><unicode>
><ctrl-z>
+CMGW: 112
 
OK
at+cmss=112
+CMSS: 5
OK
 
 
Delete message after send
 
at+cmgd=112
OK

Любую документация к комплектующим вы сможете найти на отраслевом ресурсе www.kiosksoft.ru