As of April 25, 2008 (V4.3)

TBD900 Memory Switch Table

(Star internal use only)

- Memory Switch Definition Command: (into RAM)

<ESC> <GS> # m N n1n2n3n4 <LF><NUL>

m=”,”: Define memsw data as “word set”

N: number of memsw

n1n2n3n4: defined data

m=”+”: Define memsw data “bit set”

N: number of memsw

n1n2n3n4: defined data (define as a bit to set)

m=”-“: Define memsw data to clear  “bit clear”

N: number of memsw

n1n2n3n4: defined data (define as a bit to clear)

m=”@”: Clear all memsw (set to all “0”)

N: 0

n1n2n3n4: 0000

- Memory Swtich Setting Command:

<ESC> <GS> “#W00000” <LF><NUL>
Write memsw data to Flash ROM, then reset printer

<ESC> <GS> “#T00000” <LF><NUL>

Write memsw data to Flash ROM, then reset printer and print self test.

- Memory Switch Setting Confirmation Command (hidden function):

<ESC>”#N ? 0000”<LF><NUL>

N = Memory Switch Number

[Transmission format]

<ESC> “#N , n1n2n3n4”<LF><NUL>

- ROM Name Confirmation Command (hidden function):

<ESC>”#* ”<LF><NUL>

[Transmission format]

<ESC> “#*ROM Name”<LF><NUL>

i.e. for ROM Name = TSP600 Ver1.0

<ESC> “#*,TSP600Ver1.0”<LF><NUL>

Caution: Do not use these commands per each receipt, which will shorten extremely the EEPROM life.

[1] Memory switch #0

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 | Destination | SBCS | DBCS | L/P | E | 1.0 | *1 |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Star L: Line mode

Star P: Page mode (Support Page mode from V2.0)

E: 	  ESC/POS mode (Support ESC/POS mode from V3.0)

Note *1	TBD900 supports DBCS version.

Flash ROM 8M(programI)+8M(3.5M: Logo store, 4M: DBCS Character, 0.5M: Reserved)

[2] Memory switch #1

: Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E | Paper position | See below |  | L/P | E | 1.0 | *1 |
| D C | Paper position | See below |  | L/P | E | 1.0 | *2 |
| B | Font size | Standard | Large |  | E | 3.1 | *5 |
| A |  |  |  |  |  |  |  |
| 9 | TOP search | Invalid(15mm) | Valid(3mm) | L/P | E | 1.0 | *3 |
| 8 | BM function | Invalid | Valid | L/P | E | 1.0 |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 | Zero style | Normal | Slash | L/P | E | 1.0 |  |
| 3 | International Character Set | See Below |  | L/P |  | 1.0 | *4 |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

*1: Paper position (Retrun to ON LINE)

| Presenter | BM function | MSW1-E | Paper position at recover to ON LINE |
| --- | --- | --- | --- |
| with | Valid | 0 | Detect BM + Full cut + eject |
|  | Valid | 1 | Detect BM + Full cut + eject |
|  | Invalid | 0 | Feed + Full cut + eject |
|  | Invalid | 1 | Feed + Full cut + eject |
| without | Valid | 0 | Detect BM + Full cut |
|  | Valid | 1 | Detect BM |
|  | Invalid | 0 | Feed + Full cut |
|  | Invalid | 1 | Feed + Full cut |

*2: Paper positon at POWER ON

| Presenter | BM function | MSW1-D | MSW1-C | Paper position at Power ON |
| --- | --- | --- | --- | --- |
| with | Valid | 0 | 0 | Hold |
|  |  | 0 | 1 | Detect BM + Full cut + eject |
|  |  | 1 | 0 | Reserved |
|  |  | 1 | 1 | Reserved |
|  | Invalid | 0 | 0 | Hold |
|  |  | 0 | 1 | Feed + Full cut + eject |
|  |  | 1 | 0 | Reserved |
|  |  | 1 | 1 | Reserved |
| without | Valid | 0 | 0 | Hold |
|  |  | 0 | 1 | Detect BM + Full cut |
|  |  | 1 | 0 | Detect BM |
|  |  | 1 | 1 | Reserved |
|  | Invalid | 0 | 0 | Hold |
|  |  | 0 | 1 | Feed + Full cut |
|  |  | 1 | 0 | Reserved |
|  |  | 1 | 1 | Reserved |

*3: TOP search

Invalid: TOP margin is distance between cutter and head, = 15mm

Valid:  TOP margin is 3mm.  After full cut, feed backwards approx. 13mm, and printer

receives next data, feed 1mm (back rush) and print next data. = 3mm

*4: International Character sets

| n4 | MSW1-3 | MSW1-2 | MSW1-1 | MSW1-0 | Characters |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | USA |
| 1 | 0 | 0 | 0 | 1 | France |
| 2 | 0 | 0 | 1 | 0 | Germany |
| 3 | 0 | 0 | 1 | 1 | U.K |
| 4 | 0 | 1 | 0 | 0 | Denmark #1 |
| 5 | 0 | 1 | 0 | 1 | Sweden |
| 6 | 0 | 1 | 1 | 0 | Italy |
| 7 | 0 | 1 | 1 | 1 | Spain #1 |
| 8 | 1 | 0 | 0 | 0 | Japan |
| 9 | 1 | 0 | 0 | 1 | Norway |
| A | 1 | 0 | 1 | 0 | Denmark #2 |
| B | 1 | 0 | 1 | 1 | Spain #2 |
| C | 1 | 1 | 0 | 0 | Latin America |
| D | 1 | 1 | 0 | 1 | Korean |
| E | 1 | 1 | 1 | 0 | Ireland |
| F | 1 | 1 | 1 | 1 | Legal |

* 5: Support Large Font A:  Firmware version 3.0 or higher

Support Large Font A and Font B:  Firmware version 3.1 or higher

[3] Memory switch  #2

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | Print start position adjustment direction | Forward | Backward | L/P |  | 1.0 | *1 |
| E | Print start position adjustment | See Below |  | L/P |  | 1.0 | *1 |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 | Print speed | See Below |  | L/P | E | 1.0 | *2 |
| 4 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 2 | Print density | See Below |  | L/P |  | 1.0 | *3 |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Note *1 :

| n1n2 | F - 8 | Length | n1n2 | F - 8 | Length |
| --- | --- | --- | --- | --- | --- |
| 00 | 00000000 | +0.00 mm | 80 | 10000000 | -0.00 mm |
| 01 | 00000001 | +0.25 mm | 81 | 10000001 | -0.25 mm |
| 02 | 00000010 | +0.50 mm | 82 | 10000010 | -0.50 mm |
| 08 | 00001000 | +2.00 mm | 88 | 10001000 | -2.00 mm |
| 0F | 00001111 | +3.75 mm | 8F | 10001111 | -3.75 mm |
| 10 | 00010000 | +4.00 mm | 90 | 10010000 | -4.00 mm |
| 18 | 00011000 | +6.00 mm | 98 | 10011000 | -6.00 mm |
| 20 | 00100000 | +8.00 mm | A0 | 10100000 | -8.00 mm |
| 28 | 001101000 | +10.00 mm | A8 | 10101000 | -10.00 mm |
| 30 | 00110000 | +12.00 mm | B0 | 10110000 | -10.00 mm |
| 7F | 01111111 | +31.75 mm | FF | 11111111 | -10.00 mm |

This function is enabled only when “Detect start position” (MSW1-8 = 1) is ON.

This memory switch settings precisely adjust start position that may varies depending on paper quality , darkness of printed black mark , or sensor volume setting. (Software adjusts paper feed length after detect black mark or gap between sensor and print head.)  -10.00 to +31.75 mm

Note *2: Print speed

| n3 | MSW2-5 | MSW2-4 | Speed (momo color mode) | Speed (2 color mode) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | High Speed (150mm/s) | 60mm/s |
| 1 | 0 | 1 | Middle Speed (110mm/s) | 60mm/s |
| 2 | 1 | 0 | Low Speed (60mm/s) | 60mm/s |
| 3 | 1 | 1 | Reserved | Reserved |

Note *3: Print density

| n4 | MSW2-2 | MSW2-1 | MSW2-0 | Density(momo color mode) | Density(2 color mode) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1.0 | 1.0 |
| 1 | 0 | 0 | 1 | 1.1 | 1.0 |
| 2 | 0 | 1 | 0 | 1.2 | 1.2 |
| 3 | 0 | 1 | 1 | 1.3 | 1.2 |
| 4 | 1 | 0 | 0 | 1.0 | 1.0 |
| 5 | 1 | 0 | 1 | 0.9 | 1.0 |
| 6 | 1 | 1 | 0 | 0.8 | 0.8 |
| 7 | 1 | 1 | 1 | 0.7 | 0.8 |

[4] Memory switch  #3

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | Codepage | See next page |  | L |  |  | *2 |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 | SBCS right space adjust | See below |  |  | E | 3.0 | *3 |
| 6 |  |  |  |  |  |  |  |
| 5 | DBCS characters print column | See below |  | L |  | 1.0 | *1 |
| 4 | SBCS characters print column | See below |  | L |  | 1.0 | *1 |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 | <CR> code | Invalid | Same as <LF> |  |  |  |  |
| 0 | Line spacing | 4 mm | 3 mm |  |  |  |  |

Note *1	DBCS/SBCS character print column

When the MSW 0-4 set to “0”

| MSW3-4 | Character | Horizontal size (font H size + space) | Printable area MSW4-0 to 4-2 | Print column |
| --- | --- | --- | --- | --- |
| 0 | SBCS | 12(12+0) dots | 104mm (832dots) | 69 |
|  |  |  | 80mm (640dots) | 53 |
|  |  |  | 72mm (576dots) | 48 |
|  |  |  | 56mm (448dots) | 37 |
| 1 | SBCS | 15(12+3) dots | 104mm (832dots) | 55 |
|  |  |  | 80mm (640dots) | 42 |
|  |  |  | 72mm (576dots) | 38 |
|  |  |  | 56mm (448dots) | 29 |

When the MSW 0-4 set to “1”

| MSW3-5 | Character | Horizontal size (space+ font + space) | Printable area MSW4-0 to 4-2 | Print column |
| --- | --- | --- | --- | --- |
| 0 | DBCS | 26(1+24+1) dots | 104mm (832dots) | 32 |
|  |  |  | 80mm (640dots) | 24 |
|  |  |  | 72mm (576dots) | 22 |
|  |  |  | 56mm (448dots) | 17 |
| 1 | DBCS | 30(3+24+3) dots | 104mm (832dots) | 27 |
|  |  |  | 80mm (640dots) | 21 |
|  |  |  | 72mm (576dots) | 19 |
|  |  |  | 56mm (448dots) | 14 |

| MSW3-4 | Character | Horizontal size (font + space) | Printable area MSW4-0 to 4-2 | Print column |
| --- | --- | --- | --- | --- |
| 0 | SBCS | 13(12+0) dots | 104mm (832dots) | 69 |
|  |  |  | 80mm (640dots) | 53 |
|  |  |  | 72mm (576dots) | 48 |
|  |  |  | 56mm (448dots) | 37 |
| 1 | SBCS | 15(12+3) dots | 104mm (832dots) | 55 |
|  |  |  | 80mm (640dots) | 42 |
|  |  |  | 72mm (576dots) | 38 |
|  |  |  | 56mm (448dots) | 29 |

Character table

| n1n2 | bit F | bit E | bit D | bit C | bit B | bit A | bit 9 | bit 8 | Character table |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Normal |
| 01 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | CodePage437(USA,std. Europe) |
| 02 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | Katakana |
| 03 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | CodePage437(USA,std. Europe) |
| 04 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | CodePage858 (Multilingual) |
| 05 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | CodePage852 (Latin-2) |
| 06 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | CodePage860 (Portuguese) |
| 07 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | CodePage861 (Icelandic) |
| 08 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | CodePage863 (Canadian Frence) |
| 09 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | CodePage865 (Nordic) |
| 0A | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | CodePage866 (Cyrillic Russian) |
| 0B | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | CodePage855 (Cyrillic Bulgarian) |
| 0C | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | CodePage857 (Turkish) |
| 0D | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | CodePage862 (Hebrew) |
| 0E | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | CodePage864 (Arabic) |
| 0F | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | CodePage737 (Greek) |
| 10 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | CodePage851 (Greek) |
| 11 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | CodePage869 (Greek) |
| 12 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | CodePage928 (Greek) |
| 13 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | CodePage772 (Lithuanian) |
| 14 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | CodePage774 (Lithuanian) |
| 15 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | CodePage874 (Thai) |
| 20 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | CodePage1252 (Windows Latin-1) |
| 21 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | CodePage1250 (Windows Latin-2) |
| 22 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | CodePage1251 (Windows Cyrillic) |
| 40 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | CodePage3840 (IBM-Russian) |
| 41 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | CodePage3841 (Gost) |
| 42 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | CodePage3843 (Polish) |
| 43 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | CodePage3844 (CS2) |
| 44 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | CodePage3845 (Hungarian) |
| 45 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | CodePage3846 (Turkish) |
| 46 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | CodePage3847 (Brazil-ABNT) |
| 47 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | CodePage3848 (Brazil-ABICOMP) |
| 48 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | CodePage1001 (Arabic) |
| 49 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | CodePage2001 (Lithuaninan-KBL) |
| 4A | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | CodePage3001 (Estonian-1) |
| 4B | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | CodePage3002 (Estonian-2) |
| 4C | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | CodePage3011 (Latvian-1) |
| 4D | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | CodePage3012 (Latvian-2) |
| 4E | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | CodePage3021 (Bulgarian) |
| 4F | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | CodePage3041 (Maltese) |
| FF | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | Brank CodePage |

Note *2 : When the printer is SBCS, support Code page.

DBCS: Fixed to Normal.

Note *3 : When use Large FontA (MSW1-7)

| MSW3-7 | MSW3-6 | SBCS right space adjustment |
| --- | --- | --- |
| 0 | 0 | No |
| 0 | 1 | +1 dot |
| 1 | 0 | +2 dots |
| 1 | 1 | +3 dots |

[5] Memory switch #4

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 | Raster data processing speed | Standard | High |  | E | 3.2 | *2 |
| 8 | Print mode | Uni color | Two color |  | E | 3.0 |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 2 | Printable width | See below |  | L/P |  | 1.0 | *1 |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Note *1 Number of columns         Hidden function

| n4 | bit-2 | bit-1 | bit-0 | Printable area | TypicalPaper width |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 104 mm (832dot) | 112 mm |
| 1 | 0 | 0 | 1 | 80 mm (640dot) | 82.5 mm |
| 2 | 0 | 1 | 0 | 72 mm (576dot) | 80 mm |
| 3 | 0 | 1 | 1 | 56 mm (448dot) | 58 mm |
| 4 | 1 | 0 | 0 | Reserved |  |
| 5 | 1 | 0 | 1 |  |  |
| 6 | 1 | 1 | 0 |  |  |
| 7 | 1 | 1 | 1 |  |  |

Note *2

In case of all data are “raster” data, it should better to change this memory switch.

We add this function to avoid garvage print when the printer recives raster + barcode data.

[6] Memory switch #5

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D | Presenter minimum page length | See below |  | L/P | E | 1.2 | *2 |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A | Paper adjustment direcdtion | + | - | L/P | E | 4.3 | *3 |
| 9 | Paper adjustment length | See below |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 | Presenter capture timer | See below |  | L/P |  | 1.0 | *1 |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

*1: Presenter capture timer

| n3 n4 | bit7 | bit6 | bit5 | bit4 | bit3 | bit2 | bit1 | bit0 | Capture timer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | Capture control by command |
| 0 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0.5 sec. |
| 0 2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1.0 sec. |
| 0 3 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1.5 sec. |
| 0 4 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 2.0 sec. |
| 0 8 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 5.0 sec. |
| 1 4 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 10.0 sec. |
| 2 8 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 20.0 sec. |
| 3 C | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 30.0 sec. |
| 7 8 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 60.0 sec. |
| B 4 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 90.0 sec. |
| F 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 120.0 sec. |
| F F | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 127.5 sec. |

*2: Presenter minimum page length

| bit D | bit C | Length |
| --- | --- | --- |
| 0 | 0 | 72 mm (default) |
| 0 | 1 | 100 mm |
| 1 | 0 | 130 mm |
| 1 | 1 | 150 mm |

When the page length(received data) is shorter than minimum page length set by memsw, printer

feed paper until set length.

*3: When the presenter mode is (MSW 6-3=0, 6-2=0, 6-1=0, 6-0=1: Loop – Hold (pull off) – discharge),

This function is available.

| 5-A | 5-9 | 5-8 | Paper adjustment length (mm) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Adjustment length = 0 |
| 0 | 0 | 1 | Adjustment length = 1 |
| 0 | 1 | 0 | Adjustment length = 2 |
| 0 | 1 | 1 | Adjustment length = 3 |
| 1 | 0 | 0 | Adjustment length = 0 |
| 1 | 0 | 1 | Adjustment length = -1 |
| 1 | 1 | 0 | Adjustment length = -2 |
| 1 | 1 | 1 | Adjustment length = -3 |

[7] Memory switch #6

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E | Auto loading speed | 150mm/sec | 32mm/sec | L/P | E | 4.1 |  |
| D | Auto loading retry | Valid | Invalid | L/P | E | 3.3 |  |
| C | Auto loading function | Valid | Invalid | L/P | E | 1.0 |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 | Paper in presenterat power on | Exit | Cut then exit | L/P | E | 1.2 |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 | Presenter paper position status (ASB) | Invalid | Valid | L/P | E | 1.2 | *2 |
| 3 | Presenter mode | See below |  | L/P | E | 1.0 | *1 |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Note *1: Presenter mode

| n4 | bit3 | bit2 | bit1 | bit0 | Mode |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | Loop – Hold (pull off) – capture |
| 1 | 0 | 0 | 0 | 1 | Loop – Hold (pull off) – discharge |
| 2 | 0 | 0 | 1 | 0 | w/o Loop – Hold (pull off) – capture |
| 3 | 0 | 0 | 1 | 1 | w/o Loop – Hold (pull off) – discharge |
| 4 | 0 | 1 | 0 | 0 | w/o Loop – discharge (journal mode) |

Note *2: Add Presenter paper position status into 9th bytes of ASB status.

| Bit | Content | Status 0 | Status 1 |
| --- | --- | --- | --- |
| 7 | Fixed at 0 |  |  |
| 6 | Not Used (Fixed at 0) |  | --- |
| 5 | Not Used (Fixed at 0) |  | --- |
| 4 | Fixed at 0 |  | --- |
| 3 | Presenter Paper Position See below table for details |  |  |
| 2 |  |  |  |
| 1 |  |  |  |
| 0 | Fixed at 0 |  | --- |

When the MEMSW #6 bit 4 = 0, (Invalid) then printer return above bit balues all “0”. (9th byte = 00)

| bit 3 | bit 2 | bit 1 | Presenter Paper Position |
| --- | --- | --- | --- |
| 0 | 0 | 0 | Paper position 0 State where there is not paper in presenter |
| 0 | 0 | 1 | Paper position 1 State where there is suppolied (loop state) |
| 0 | 1 | 0 | Paper position 2 State (Reserved) |
| 0 | 1 | 1 | Paper position 3 State where there is discharged (Can be pulled out) |
| 1 | 0 | 0 | Paper position 4 State (Reserved) |
| 1 | 0 | 1 | Paper position 5 State (Reserved) |
| 1 | 1 | 0 | Paper position 6 State where there is recovered |
| 1 | 1 | 1 | Paper position 7 State where there is pulled out |

[8] Memory switch #7

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C | ASB function | Invalid | Valid | L/P |  | 1.0 |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 | NSB function | Invalid | Valid | L/P |  | 1.0 | *1 |
| 7 | Advanced ASB | See below |  | L |  | 1.1 | *2 |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Note *1: Parallel I/F only

Note * 2: Because of patent issue, we delete “Advanced ASB function” from V1.1.

[9] Memory switch #8

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 | Format | See below |  | P |  | 2.0 | *1 |
| 0 |  |  |  |  |  |  |  |

Note *1: Page mode load format number when power on

| Bit1 | Bit 0 | Function |
| --- | --- | --- |
| 0 | 0 | Not specified |
| 0 | 1 | Load Format 1 |
| 1 | 0 | Load Format 2 |
| 1 | 1 | Reserved |

[10] Memory switch  #9

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | Print start position adjustment direction | Forward | Backward | OK | NG | 3.1 | *1 |
| E | Print start position adjustment | See Below |  | OK | NG | 3.1 | *1 |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 | Black Mark length | See Below |  | OK | NG | 3.1 | *2 |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Note * 1: V3.1 or higher

This function is shifted from #2.

Forward direction

Adjustment length = Adjustment value x 0.5mm (Max. 1023.5mm)

Reverse direction

Adjustment length = Adjustment value x 0.5mm (Max. 10.0mm)

Forward direction (MSW 9-F = OFF)		　Reverse direction (MSW 9-F = ON)

| n | MSW9-F to MSW9-4 | Length |  |  | MSW9-F to MSW9-4 | Length |
| --- | --- | --- | --- | --- | --- | --- |
| 000 | 000000000000 | +0.0mm |  | 800 | 100000000000 | -0.0mm |
| 001 | 000000000001 | +0.5mm |  | 801 | 100000000001 | -0.5mm |
| 002 | 000000000010 | +1.0mm |  | 802 | 100000000010 | -1.0mm |
| 003 | 000000000011 | +1.5mm |  | 803 | 100000000011 | -1.5mm |
| ： | ： | ： |  | ： | ： | ： |
| 008 | 000000001000 | +4.0mm |  | 808 | 100000001000 | -4.0mm |
| ： | ： | ： |  | ： | ： | ： |
| 010 | 000000010000 | +8.0mm |  | 810 | 100000010000 | -8.0mm |
| ： | ： | ： |  | ： | ： | ： |
| 018 | 000000011000 | +12.0mm |  | 818 | 100000011000 | -10.0mm |
| ： | ： | ： |  | ： | ： | ： |
| 020 | 000000100000 | +16.0mm |  | 820 | 100000100000 | -10.0mm |
| ： | ： | ： |  | ： | ： | ： |
| 028 | 000000101000 | +20.0mm |  | 828 | 100000101000 | -10.0mm |
| ： | ： | ： |  | ： | ： | ： |
| 030 | 000000110000 | +24.0mm |  | 830 | 100000110000 | -10.0mm |
| ： | ： | ： |  | ： | ： | ： |
| ： | ： | ： |  | ： | ： | ： |
| 7FF | 011111111111 | +1023.5mm |  | FFF | 111111111111 | -10.0mm |

Note *2: Black Mark length, V3.1 or higher

When the Black Mark length is longer than 9mm, printer goes to paper end error in default

setting.  This function support longer Black Mark length paper.

Black Mark length = (Black Mark adjust value) x 1mm (9mm <= BM length <=24mm)

| n | MSW9-3 to MSW9-0 | BM Length |
| --- | --- | --- |
| 0 | 0000 | 9mm |
| 1 | 0001 | 10mm |
| 2 | 0010 | 11mm |
| 3 | 0011 | 12mm |
| 4 | 0100 | 13mm |
| 5 | 0101 | 14mm |
| ： | ： | ： |
| 15 | 1111 | 24mm |

[11] Memory switch #A

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C | Reserved |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A | NE sensor singal Level | NE: High | NE: Low | L/P | E | 1.1 | *3 |
| 9 | NE sensor | Photo | Mechanical | L/P | E | 1.1 | *4 |
| 8 | Paper Near End | Valid | Invalid | L/P |  | 1.0 | *1 |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 | Treatment of PE signal | Paper End | Paper End or Near End | L/P |  | 1.0 | *2 |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Note *1: Paper Near End

This function is available when the DIPSW 1-8 set to OFF (Near End Sensor is connected.)

Note *2: Treatment of PE signal

This function is available when the Near End sonsor is installed and I/F is parallel.

(DIPSW 1-8 set to OFF.)

bit 8 = “0”: PE signal is used only Paper End signal.

bit 8 = “1”: PE signal (parallel I/F, PError signal – pin#12) is used when the printer goes to

paper near end.

Note *3: This function is available when the DIPSW 1-8 set to OFF (Near End Sensor is connected.)

Note *4: This function is available when the DIPSW 1-8 set to OFF (Near End Sensor is connected.)

[12] Memory switch #B

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C | Data treatment recover from error | Print data | Delete data | P |  | 2.0 | *3 |
| B |  |  |  |  |  |  |  |
| A | Received buffer size | See below |  | L/P | E | 1.0 | *1 |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 | Data pool mode | Invalid | Valid | L | E | 2.0 | *4 |
| 0 | Communication error | Error | Ignore | L |  | 1.0 | *2 |

Note *1: Received buffer size

| n2 | bitA | bit9 | bit8 | Received buffer size |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 8K bytes |
| 1 | 0 | 0 | 1 | 64 bytes |
| 2 | 0 | 1 | 0 | 128 bytes |
| 3 | 0 | 1 | 1 | 256 bytes |
| 4 | 1 | 0 | 0 | 512 bytes |
| 5 | 1 | 0 | 1 | 1K bytes |
| 6 | 1 | 1 | 0 | 2K bytes |
| 7 | 1 | 1 | 1 | 4K bytes |

Note *2: Communication error treatment

bit 0 = 0 :

When the printer detects communication error, prints “?” symbol and goes to error condition.

bit 0 = 1 :

When the printer detects communication error, ignore error data.

Note *3: When the recover from error, according to this setting, print or delete data in buffer.

Note *4: Data pool mode

Valid:  While printer receives data, printer do not print data, just store data into received buffer

if printer do not receive data several time, printer treat that host stop to send data, then start to print data. When printer receives more data than received buffer size, printer goes to buffer full,

then host stops to send data, “buffer full print” occurs.

[13] Memory switch #C

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C | IEEE1284 reverse mode | Valid | Invalid | L/P | E | 1.0 | *1 |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 | ACK pulse width | 1 usec | 9 usec | L/P | E | 1.0 | *2 |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 | Treatment of missing data during reverse mode | Store | Delete | L/P |  | 1.0 | *3 |
| 3 |  |  |  |  |  |  |  |
| 2 | USB substitute Device ID | Invalid | Valid | L/P | E | 1.0 | *5 |
| 1 | USB serial number | Invalid | Valid | L/P | E | 1.0 | *6 |
| 0 | IEEE1284 Device ID | Invalid | Valid | L/P | E | 1.0 | *4 |

Note *1, *2, *3, *4

Parallel I/F only

Note *5, *6

USB I/F only

[14] Memory switch #E

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 | Partial cut adjust direction | + | - | L/P | E | 3.1 | *1 |
| 3 | Partial cut remaining length adjustment | See below |  | L/P | E |  | *1 |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

Note*1 : Adjust partial cut remaining length controlled by DC motor driven time.

| n4 | bit3 | bit2 | bit1 | bit0 | Remaining (+) | Remaining (-) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 msec | 0 msec |
| 1 | 0 | 0 | 0 | 1 | 1 msec | - 1msec |
| 2 | 0 | 0 | 1 | 0 | 2 msec | -2 msec |
| 3 | 0 | 0 | 1 | 1 | 3 msec | -3 msec |
| 4 | 1 | 1 | 0 | 0 | 4 msec | -4 msec |
| 5 | 1 | 1 | 0 | 1 | 5 msec | -5 msec |
| 6 | 1 | 1 | 1 | 0 | 6 msec | -6 msec |
| 7 | 1 | 1 | 1 | 1 | 7 msec | -7 msec |
| 8 | 1 | 0 | 0 | 0 | 8 msec | -8 msec |
| 9 | 1 | 0 | 0 | 1 | 9 msec | -9 msec |
| A | 1 | 0 | 1 | 0 | 10 msec | -10 msec |
| B | 1 | 0 | 1 | 1 | 11 msec | -11 msec |
| C | 1 | 1 | 0 | 0 | 12 msec | -12 msec |
| D | 1 | 1 | 0 | 1 | 13 msec | -13 msec |
| E | 1 | 1 | 1 | 0 | 14 msec | -14 msec |
| F | 1 | 1 | 1 | 1 | 15 msec | -15 msec |

[15] Memory switch #F

Hidden function

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 | Customize | See below |  | L/P | E |  | *1 |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

| Bit 3 | Bit 2 | Bit 1 | Bit 0 | Customize | Firmware Version |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Standard | 1.0 |
| 0 | 0 | 0 | 1 | AI Technology | 2.0 |
| 0 | 0 | 1 | 0 | Reserved |  |
| 0 | 0 | 1 | 1 |  |  |
| 0 | 1 | 0 | 0 |  |  |
| 0 | 1 | 0 | 1 |  |  |
| 0 | 1 | 1 | 0 |  |  |
| 0 | 1 | 1 | 1 |  |  |
| 1 | 0 | 0 | 0 |  |  |
| 1 | 0 | 0 | 1 |  |  |
| 1 | 0 | 1 | 0 |  |  |
| 1 | 0 | 1 | 1 |  |  |
| 1 | 1 | 0 | 0 |  |  |
| 1 | 1 | 0 | 1 |  |  |
| 1 | 1 | 1 | 0 |  |  |
| 1 | 1 | 1 | 1 |  |  |

Note*1: Customize for AI Technology (SMA)

i) Modify ASB status function as followings.

| ASB | bytes | TUP900 Standard | AI Technology |
| --- | --- | --- | --- |
| Number of bytes | - | 9 bytes | 7 bytes |
| Headder 1 | The first byte | 0x23 (ASB: 9 bytes) | 0x0f (ASB: 7 bytes) |
| Headder 2 | The second byte | 0x06 (ASB version 3) | 0x02 (ASB version 1) |
| Cover Open | Fifth bit of the third byte | Valid | Invalid (Fixed to “0”) |
| Head Up | First bit of the fifth byte | Invalid (Fixed to “0”) | Valid (Cover open) |
| Presenter status | First bit of the seventh byte | Invalid (Fixed to “0”) | Valid |

ii) Presenter control

When the paper is in the presenter, printer ignore <ESC> “@” command

[16] Memory switch #U

| bit | Function | 0 | 1 | Star | ESC/POS | Ver. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F | User defined number area |  |  | L/P |  | 1.0 |  |
| E |  |  |  |  |  |  |  |
| D |  |  |  |  |  |  |  |
| C |  |  |  |  |  |  |  |
| B |  |  |  |  |  |  |  |
| A |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |

You can set from “0000” to “FFFF” as a user defined number in this #.

Please note that when you reset memsw to all “0”, this switch also cleared.