ROM history:  TBD900

DIPSW2-1: OFF(flash rewrite mode)

DIPSW2-2: Rewrite area    ON: Main program          OFF: Font

DIPSW2-3: Boot rewirte     ON: Invalid	           OFF: Valid

| Date | Name | Ver. | Boot | ECN | Contents |
| --- | --- | --- | --- | --- | --- |
| ’02.06.06 | TB9 | 1.0 | 1.0 | TUP9-011 | New |
| ’02.08.12 |  | 1.1 | 1.0 | TUP9-021 | Change NE sensor specification 1. MSW#A bit A NE sensor input singal level bit A = ”0” : Detect NE, then goes to High bit A = “1” : Detect NE, then goes to Low bit 9 NE sensor bit 9 = “0” : Use Photo sensor as a NE sesor bit 9 = “1” : Use Mechanical sensor as a NE sensor 2. Add NE sensor adjust mode (DIPSW & operation panel) 3. Support Ethernet I/F (note: Firmware side only, do not finish EMI/EMC check,yet) |
| ’02.11.13 |  | 1.2 | 1.1 | TUP9-030 | New USB support (have to reflash boot program) When connect to New USB I/F, then DIPSW function change as followings DIPSW1-5; DIPSW1-6; Function. ON; ON; Mode 0. OFF; ON; Mode 2. ON; OFF; Mode 1. Mode 0: Printer Class (Same function as existing USB I/F) Mode 1: Vendor Class (New function to support OPOS) Mode 2: Printer Class (New function) USB seirial number resiter command <ESC> “## W 8, d1 d2 …. d8” <LF> <NUL> d1 d2 …. d8: serial number 48<=d<=57 (“0” <=d<=”9), 65<=d<=90 (“A”<=d<=”Z”) d= 63 (“?”), then clear serial number. ASB function (See memsw table V1.2 for details) Change ASB status bytes from 8 bytes to 9 bytes ASB header 1: Old: 0x21 New: 0x23 ASB header 2: Old: 0x04 New: 0x06 9th bytes: Presenter paper position status Memsw #6 bit 4 = “0”: Invalid presenter paper position status bit 4 = “1”: Valid presenter paper position status (See Status Function manuals for details.) At power on, paper in the presenter, (request from Mecaf) Memsw #6, bit 8 = “0”: Exit paper (default) bit 8 = “1”: Cut then exit paper Set minimum page length (request from Mecaf) Memsw #5, bit C = “0”, bit D = “0” :72mm (default) bit C = “1”, bit D = “0” : 100mm bit C = “0”, bit D = ”1” : 130mm bit C = “1”, bit D = “1” : 150mm While the paper in presenter, printer ignore peripheral drive commands, and buzzer command. (compatibility with TUP400) |
| ’03.02.06 |  | 2.0 | 1.1 | TUP9-042 | Add Star Page Mode (DIPSW 1-1:OFF, 1-2:ON) Add customize for AI Technology mode. |
| ’03.04.15 |  | 2.1 | 1.1 | TUP9-055 | Add presenter sensors adjustment mode for production line purpose. You do’t need to adjust these sensor by your side. |

| Date | Name | Ver. | Boot | ECN | Contents |
| --- | --- | --- | --- | --- | --- |
| ’03.05.13 | TB9 | 3.0 | 1.1 | TUP9-062 | Add ESC/POS mode (DIPSW 1-1:OFF, 1-2:OFF) * Target:: BA-T500 * I/F: Parallel, Serial, USB(Star original), Ethernet (Star original) USB: Star ASB stauts Ethernet: Star ASB + ESC/POS ASB- Ethernet Ver.2 only- * Black Mark support * 2 color support (Star original command & memsw settting) Memsw 4-8 set to OFF (Two color mode) <GS> “( N” pL pH n m pL=2, pH=0, n=48 m=49: Black (high energy) – default m=50: Red (low energy) * Maintenance counter (Same command as Star mode) * Presenter control command (Same command as Star mode) Support Ethernet V2.0 * When peper near end, it can send message (V1- when the paper end) * Support <ENQ>, <EOT> or other enquiry command * Web can display “Presenter counter” value * Reset by printer |
| ’04.01.09 |  | 3.1 | 1.1 | TUP9-075 | - Support PDF417 (Star Line/Page, ESC/POS mode) - Cetrificated GB2312 font (T9#CH 1.1) - Support Korean font as official version - Support Taiwan font as official version (full size support) * Can not use Logo Store function - Support Large FontB * Memsw #1 bit 7 = 1: Valid Large Font A and B - Support reset by switch combination Press Error SW and Feed SW - While ON line - While OFF line (Paper end, cover open) - While unrecoverable error - Improve BM adjustment (same as TSP700 V5.0) Memsw #9 - Support variable size of Black Mark length (same as TSP700 V5.0) Memsw #9 - Partial cut remaining length adjustment Memsw #E |
| ’04.04.13 |  | 3.2 | 1.1 | TUP9-080 | ESC/POS bug. When the printer received raster and barcode data, in some timing, printer do not treat raster data correctly, print garbage data, to avoid this bug, we add memsw function select raster data receive speed “Standard” and “High speed”. MEMSW #D bit 4 = 0: Standard (default) bit 4= 1: High speed In case of print data is all “raster”, this bug do not occur, so change memsw #D bit 4 = 1. |
| ’04.12.17 |  | 3.3 | 1.1 | TUP9-096 | Add retry function when auto loading failure Default: Valid (memsw #6-D = “0” Valid, “1” Invalid) Add following two commands for Longhorn printer driver support Print area set command <ESC> <RS> “A” n BM Valid/Invalid command <ESC> <RS> “m” n Debug Font B, Codepage #1001 (Arabic) font. Same font as FontA |
| ’06.03.07 |  | 4.0 | 2.0 | TUP9-105 | Change DeviceID for Windows Vista inbox driver support V1.0 – V3.3 TUP942 MDL: TUP900 TUP992 MDL: TUP900 V4.0 or higher TUP942 MDL: TUP900 TUP992 MDL: TUP992 Windows2K-XP, 9x drivers alredy supported New Device ID from ’04 Oct. 18 version Change Codepage874 character 0x80, from Euro to Thai font. Support real time status command even paper in the presenter. <ESC> <ACK> <SOH>, <ENQ>, <EOT> (Star Line, Page mode) |
| Date | Name | Ver. | Boot | ECN | Contents |
| ’07.01.10 | TB9 | 4.1 | 2.0 | TUP9-110 | Add autoloading speed selection by memsw MSW6-E=0 (Default) 150mm/sec MSW6-E=1 32mm/sec |
| ’07.01.25 |  | 4.2 | 2.0 | TUP9-111 | Support reflash w/o DIPSW change. Need to use MOT2FMEM V2.2.0.1 or higher. |
| ’08.04.10 |  | 4.3 | 2.0 | TUP9-121 | - Countermeasure of presenter error problem Presenter Error occurs when the following condition - Print speed = Low speed (60mm/sec) - Presenter mode = Eject Mode - “Curly“ paper To avoid this problem, Printer setting is 60mm/sec, change feed length between sensor 1 to sensor 2, from 20mm to 22mm. And add feed length adjustment function in msw. MSW5-A Adjustment direction 5-A=0: +, 5-A=1: - (minus) MSW5-9=0, 5-8=0: 0mm (default) MSW5-9=0, 5-8=1: 1mm MSW5-9=1, 5-8=0: 2mm MSW5-9=1, 5-8=1: 3mm - Add Real Time Reset Command in Line Mode <ESC> <ACK> <CAN> - Add “Presenter initialization at Power ON“ MSW6-9=0, 6-8=0: Eject (default) |
| ’09.10.20 |  | 4.4 | 2.0 | TUP9-156 | Counter mesure for presenter sensor mulfunction when use out of spec. Paper. (pre-printed paper) (apply from Nov. Production) |